# High Fidelity Speech Enhancement with Band-split RNN

Jianwei Yu*, Hangting Chen* , Yi Luo, Rongzhi Gu, Chao Weng

Tencent AI Lab, Audio and Speech Signal Processing Oteam

{tomasyu,  oulyluo,  erichtchen,  lorrygu,  cweng}@tencent .com

Abstract

arXiv:2212.00406v2  [ee ss.AS]  6 Jun 2023

Despite  the  rapid  progress  in  speech  enhancement  (SE)  re- search, improving the intelligibility and perceptual quality of desired speech in noisy environments with interfering speak- ers remains challenging.  This paper attempts to achieve high- fidelity full-band SE and personalized SE (PSE) by modifying the recently proposed band-split RNN (BSRNN) model.   To reduce the negative impact of unstable high-frequency compo- nents in full-band speech recording, we perform bi-directional and uni-directional band-level modeling to low-frequency and high-frequency subbands, respectively.   For the PSE task, an additional speaker enrollment module is added to BSRNN to make use of the target speaker information for suppressing the interfering  speech.   Moreover,  we  utilize  a  MetricGAN  dis- criminator (MGD) and a multi-resolution spectrogram discrim- inator (MRSD) to further improve the human auditory percep-

# tual quality of the enhanced speech. Experimental results show

that our system outperforms various top-ranking SE systems, achieves state-of-the-art (SOTA) SE performance on the DNS- 2020 test set, and ranks among the top 3 in the DNS-2023 chal- lenge on the PSE task.

Index  Terms—   Band-split  RNN,  Full-band  speech  en- hancement, Personalized speech enhancement

# 1.  Introduction

Speech enhancement (SE) is an important task in speech com- munication that aims to improve the subjective and objective quality of speech signals.  Such improvements can not only be helpful for human listeners to better understand the contents but also be beneficial for machine listeners to generate more ac- curate transcriptions.  In recent few years, deep learning based SE models have made a great progress due to the emergence of novel model architectures  [1–6],  efficient  data  simulation and model training pipelines [7, 8], and international challenges [9, 10] with comprehensive evaluation metrics and large-scale training data. Due to the rising demand for high-fidelity (Hi-Fi) speech in online conferencing systems and high-definition live streaming, recent developments in speech enhancement models have attempted to handle super wide-band (24 kHz) and full- band (48 kHz) speech signals [11, 12].  Moreover, since con- ventional SE models cannot suppress the speech of interfering speakers, personalized SE (PSE) models have also been pro- posed [13–16] to isolate the target speaker’s voice from inter- fering speech using target speaker’s enrollment speech.

Although previous SE and PSE models can largely improve the quality of the desired speech signals, the performance of

* Equal contribution

these models on full-band signals is restricted by the follow- ing two  aspects.   First,  in real-time  speech  communication, different devices have various effective sample rates and fre- quency responses.   Some  devices can precisely capture rela- tively high-frequency (>8 kHz) information of the speech sig- nal,  while  others may introduce  significant  distortion in the high-frequency range.    Such unpredictable  device-dependent distortion on high-frequency components can degrade the per- formance of full-band speech enhancement.  Second, the con-

# ventional training objectives of SE models, such as signal-to-

noise ratio  (SNR)  and  frequency-domain  mean  square  error (MSE), are not closely related to human auditory perception and other objective perceptual metrics, which can lead to sub- optimal SE performance.

This paper aims to tackle the problems mentioned above by applying the band-split RNN (BSRNN) model, which was recently introduced in [3], to full-band SE and PSE tasks.  In the modified BSRNN, the input signal’s spectrogram is initially divided into a collection of frequency bands, which are then converted into subband features using band-specific fully con- nected (FC) layers.  For full-band SE and PSE tasks, we per- form bi-directional band-level modeling for subbands below 8 kHz and uni-directional modeling for subbands above 8 kHz to mitigate the effect of high-frequency distortion introduced by the devices.   For PSE task, an additional speaker enrollment module is added to BSRNN to make use of the target speaker information. In addition, we use a modified MetricGAN-based training objective [8, 17] to directly optimize the model towards the widely-used perceptual evaluation of speech quality (PESQ) score. Furthermore, to improve the human auditory perceptual quality of the enhanced speech, a multi-resolution spectrogram discriminator (MRSD) [18] is also adopted in model training. Experimental results show that our BSRNN system outperforms various  top-ranking  benchmark  systems  in  SE  and  achieves SOTA results on the DNS-2020 non-blind test set in both of- fline and online scenarios. For the PSE task, the proposed sys- tem achieves better PDNSMOS scores on the 4th DNS-2022 blind test set than the previous SOTA system and ranks among the top 3 in the DNS-2023 challenge on both speaker phone and headset tracks.

This paper is an extension of our two-page grand challenge report [19] for ICASSP DNS 2023 Challenge. Both online and offline systems for SE and PSE tasks are considered in this pa- per, while the two-page grand challenge report only focuses on the online PSE task. More details of the model design, training objectives, and experimental results are added in this paper.

The paper is organized as follows:  Section 2 presents the proposed BSRNN architecture. The training objectives are dis- cussed in Section 3.  Section 4 introduces the data simulation and training configurations.  Experimental results and analysis

| Band split |  | Norm + FC Norm + FC Norm + FC | (A) Band split module Merge |  | Merge Merge MLP MLP (C) Estimation module |
| --- | --- | --- | --- | --- | --- |
| RNN across K RNN across T LSTM LSTM CAT |  |  |  | (B) Band and sequence modeling module |  |
| BLSTM Repeat along T |  |  |  |  |  |
| Enrollment speech Speaker Embedding Model |  | Conv1D | (D) Speaker enrollment module |  |  |

Fig. 1. The diagram of the proposed full-band PSE system. (A) The band split module. (B) The sequence and band modeling module.

(C) The estimation module. (D) The speaker enrollment module.

can be found in Section 5, and Section 6 offers the conclusion.

2.  Model Architecture

The proposed system shown in Figure 1 is a frequency-domain model composed of a band split module, a band and sequence modeling module, a mask estimation module, and an additional speaker enrollment module for PSE purposes. As shown in Fig- ure 1 (A), the band split module explicitly splits the complex- valued spectrogram of the noisy input X into a set of non- overlapped frequency bands {Bi  subband features {Zi  . Resid- ual long short-term memory networks (LSTMs) are utilized in the band and sequence modeling module to conduct interleaved modeling at both the band and sequence levels of the subband features.

As discussed in Section 1, different recording devices can introduce unpredictable distortions and information lost in high- frequency components (normally >8 kHz) due to different fre- quency responses.   For  example,  the  frequency  components above 16 kHz are missing in the spectrogram shown in Figure 1 (A). Hence, the bi-directional intra-band modeling used in orig- inal BSRNN model [3] can be sub-optimal, as it can propagate high-frequency  distortions  to  lower  frequencies  and  thereby compromise the overall enhancement performance.  To atten- uate the effects of the high-frequency distortions in full-band

enhancement, we split the subband features ˆZ to  ˆZ1  and  ˆZ2 ,

where  ˆZ1  contains the subband features below 8 kHz and  ˆZ2

contains subbands above 8 kHz. Then we perform bi-directional

band-level modeling on  ˆZ1 and uni-directional modeling on  ˆZ2

as follows:

Q1, (h, c) = BLSTM(ˆZ1 );   Q2  = LSTM(ˆZ2, (h, c))  (1)

where h and c are the hidden and cell state of the low-to-high direction obtained from the band-level modeling of ˆZ1 .  In the rest of the paper, we will use BSRNN-S to denote the BSRNN model with split band-level modeling.

After band and sequence modeling, the estimation module uses band-specific MLPs to predict the complex-value T-F mask M ∈ CF ×T .  Regarding the artifacts brought by the complex

masks, an MLP is additionally used to directly predict the tar- get speech’s complex-valued residual spectrogram R ∈ CF ×T . The final enhanced speech is obtained by:

S- = M ⊙ X + R                            (2)

where ⊙ denotes the complex-valued Hadamard product and S- represents the enhanced spectrogram.

In PSE task, a speaker enrollment module is utilized to ex- tract the target speaker information from the enrollment speech to eliminate interfering  speech.   Specifically,  the  enrollment speech is a pre-recorded 5 to 10 seconds clean or noisy speech segment from the target speaker, which is used to compute the speaker embedding e ∈ RF .  In the proposed model, we adopt a pretrained speaker embedding model [20] based on ResNet34 with frozen model parameters to compute the speaker embed- ding e. The speaker representation E ∈ RK×T × N  is then ob- tained by transforming the speaker embedding e using band- specific 1-D convolutional layers with Tanh activation function and repeated across the time axis.  Then, we concatenate the subband feature Z and the speaker representation E along the feature dimension to apply band and sequence-level modeling. Throughout the remainder of the paper, we will refer to the per- sonalized BSRNN model as pBSRNN.

3.  Training Objectives

The training objective of our model contains three parts.

Multi-resolution Frequency Loss:  The  first part is a multi- resolution (MR) frequency-domain loss LMR  designed by com- bining  the  power-compressed  amplitude  mean  average  error (MAE) loss and the complex-valued MAE loss with short-time Fourier transform (STFT) windows ranging from 10 ms to 40 ms:

where S-i  and Si  denote the complex spectra of the estimated and target speech signal, respectively, i is the index of the STFT window size from [10, 20, 30, 40] ms.  We use p = 0.3 in our implementation.

MetricGAN: To directly improve the system performance on the  PESQ  score,  adversarial  training  based  on  MetricGAN (MGD) [8,17] is adopted. The generator of MGDis the BSRNN model and the discriminator in MGD D(·) attempts to predict the normalized PESQ score:

Note that the wide-band PESQ score is used in all experiments. Two least-square GAN (LSGAN) [21] style training objectives are then designed for the generator and discriminator:

LMdGD  = (1 − D(S , S))2  + (QPESQ (S- , S) − D(S- , S))2    (5) + (QPESQ (X , S) − D(X , S))2

LMgGD  = (1 − D(S- , S))2  + 0.5 · LMR                                                (6)

where LMgGD , LMdGD  denote the generator loss and the discrimina- tor loss, respectively.  Note that the discriminator loss contains three signal pairs from noisy input, clean target and estimated target to stabilize the training process.

Multi-resolution Spectrogram Discriminator: To further en- hance the perceptual quality of the output speech, we also adopt the multi-resolution spectrogram discriminator (MRSD) [18] in model training:

where LMdRSD , LMgRSD  are the discriminator and generator objec- tive, respectively.  LMMEL  is the mean  square error (MSE) be- tween the mel-spectrograms of the target and estimated speech signals with the number of mel filterbanks ranging from [64, 128, 256]. 6 STFT windows ([2, 4, 8, 16, 32, 64] ms) denoted by the STFT window index k are used in our implementation.

4.  Experiment Configurations

4.1.  Data

Our experiments are conducted on the publicly available DNS- 2023 challenge dataset.  The training data contains ~760 hour clean speech with speaker identity labels, ~181 hour noise and ~60k room impulse response (RIR) samples. We use the DNS- 2020 and DNS-2022 blind test set for evaluation.

Preprocessing: During the experiment, we found that the orig- inal single-speaker speech utterances may contain interfering speech from other speakers. The existence of interfering speech can confuse the PSE model in training and result in sub-optimal performance. To address this issue, we divide the original utter- ances into 3-second segments and employ a pre-trained speaker embedding  model  to  detect  the  interfering  speech.    This  is achieved by computing the cosine similarity of speaker embed- dings between each segment and the enrollment speech.  We removed segments with a similarity lower than 0.6 to clean up the data.

Simulation: During training, the noisy speech signal is stimu- lated in an on-the-fly manner.  Specifically, we first sample one target speech and one noisy segment and then mix them with an SNR sampled from [-5, 20] dB. Note that we convolve 20% of the speech segments with randomly selected RIR to simulate reverberation.  For the PSE task, we additionally sample and mix an interfering speech segment with SIR ranging from [-5,

20] dB in simulation. In PSE model training, 50% of the train- ing inputs are the mixture of target speech and noise segments, 30% of the training inputs are the mixture of target, interfering speech and noise segments and 20% of the training inputs are the mixture of target and interfering speech segments.  All the simulated training segments are set to be 6-second long in our implementation.

4.2.  Model

Band-spilt  Scheme:  We  split the  spectrogram into  33  sub- bands, including twenty 200 Hz bandwidth subbands for low frequency followed by six 500 Hz subbands and seven 2 kHz subbands.

Hyperparameters:  We use the Hanning analysis window for STFT and set the window length and shift to 20 ms and  10 ms for the 48 kHz model,  and  32 ms  and  8 ms for the  16 kHz model.   We  set the feature dimension to N  =  96 and N  =  128 for the 48 kHz  and  16 kHz models,  respectively. A six-layer band and sequence modeling module with 192 di- mensional LSTM is used in our system.  The estimation mod- ule uses the 384-dimensional MLP with Tanh activation func- tion and a gated linear unit (GLU) [30] output layer.  We use layer normalization [31] for offline configuration and batch nor- malization [32] for online configuration.  The online pBSRNN model has a computational complexity of approximately 14.7G multiply-accumulate operations (MACs) per second. Using the ONNX runtime implementation on an Intel i5 2.50GHz CPU, the online PSE model achieves 0.42 real-time factor(RTF).

Speaker Embedding Model:  The pretrained speaker embed- ding model [20, 33]1  is used in our PSE system.

4.3.  Training Pipeline

To perform non-personalized speech enhancement, we train the BSRNN model (without the speaker enrollment module) from scratch using the MR loss with an initial learning rate of 1e-3 for 400k iterations. Then, we finetune the model with the MGD and MRSD training objectives for another 100k iterations. The training of PSE model is similar to the non-personalized model, and the only difference is that the PSE model uses the parame- ters of the pretrained non-personalized model as initialization.

All the models are trained on 8 Nvidia P40 GPUs using Adam optimizer with 0.98 learning rate decay for every 20k up- dates. The model training will be stopped if the best validation result is not achieved in 20k consecutive iterations.

5.  Experimental Results

5.1.  Non-personalized speech-enhancement

Table   1  compares  the  performance  between   the  proposed non-personalized SE BSRNN models and several top-ranking benchmark models on the non-blind test of DNS-2020 chal- lenge.    It  can be  observed that:   1)  Compared  with  models trained  using  wide-band  (16 kHz)  data,  the  performance  of models trained on full-band data with bi-directional band-level modeling degrades significantly (ID 6 v.s.  7, ID 16 v.s.   17), which  supports  the  claim  that  the  high-frequency  distortion can degrade the full-band enhancement performance; 2) Using bi-directional and uni-directional band-level modeling on low- frequency and high-frequency subbands separately can improve the performance of full-band SE models (ID 7 v.s.   8, ID  17

1 https://github.com/wenet-e2e/wespeaker/blob/ master/docs/pretrained .md

Table 1. Results on DNS-2020 no reverb / with reverb test sets. ‘-16k’ denotes model trained using 16 kHz data.

| ID | Model | Causal | no reverb |  |  |  | with reverb |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PESQ-WB | PESQ-NB | SI-SNR | STOI | PESQ-WB | PESQ-NB | SI-SNR | STOI |
| 1 | Noisy | − | 1.58 | 2.45 | 9.1 | 91.5 | 1.82 | 2.75 | 9.0 | 86.6 |
| 2 | SN-NET [22] | X | 3.39 | − | 19.5 | − | − | − | − | − |
| 3 | DPT-FSNET [23] | X | 3.26 | − | 20.4 | 97.7 | 3.53 | − | 18.1 | 95.2 |
| 4 | TridentSE [24] | X | 3.44 | − | − | 97.9 | 3.50 | − | − | 95.2 |
| 5 | MTFAA-NET [12] | X | 3.52 | 3.76 | − | − | − | − | − | − |
| 6 | BSRNN-16k | X | 3.45 | 3.87 | 21.1 | 98.3 | 3.72 | 4.03 | 19.1 | 96.3 |
| 7 | BSRNN | X | 3.32 | 3.79 | 21.1 | 98.0 | 3.54 | 3.94 | 18.5 | 95.6 |
| 8 | BSRNN-S | X | 3.42 | 3.85 | 21.3 | 98.3 | 3.61 | 3.95 | 19.0 | 95.8 |
| 9 | BSRNN-S + MGD | X | 3.50 | 3.85 | 21.4 | 98.4 | 3.63 | 3.95 | 19.0 | 96.1 |
| 10 | BSRNN-S + MRSD | X | 3.53 | 3.89 | 21.4 | 98.4 | 3.68 | 3.98 | 19.2 | 96.3 |
| 11 | CTS-Net [25] | ✓ | 2.94 | 3.42 | 18.0 | 96.7 | 3.02 | 3.47 | 15.6 | 92.7 |
| 12 | GaGNet [5] | ✓ | 3.17 | 3.56 | 18.9 | 97.1 | 3.18 | 3.57 | 16.6 | 93.2 |
| 13 | MDNet [26] | ✓ | 3.18 | 3.56 | 19.2 | 97.2 | 3.24 | 3.59 | 16.9 | 93.6 |
| 14 | FRCRN [27] | ✓ | 3.23 | 3.60 | 19.8 | 97.7 | − | − | − | − |
| 15 | MTFAA-NET [12] | ✓ | 3.32 | 3.63 | − | − | − | − | − | − |
| 16 | BSRNN-16k | ✓ | 3.23 | 3.73 | 19.9 | 97.7 | 3.37 | 3.82 | 17.6 | 94.8 |
| 17 | BSRNN | ✓ | 3.14 | 3.63 | 19.3 | 97.4 | 3.13 | 3.64 | 16.8 | 93.9 |
| 18 | BSRNN-S | ✓ | 3.26 | 3.73 | 20.0 | 97.6 | 3.30 | 3.77 | 17.5 | 94.5 |
| 19 | BSRNN-S + MGD | ✓ | 3.27 | 3.73 | 20.1 | 97.7 | 3.33 | 3.78 | 17.4 | 94.5 |
| 20 | BSRNN-S + MRSD | ✓ | 3.32 | 3.77 | 20.5 | 97.8 | 3.37 | 3.80 | 17.9 | 94.7 |

Table 2. PDNSMOS P.835 scores on the DNS-2022 blind test set, where SIG means speech quality, BAK means background noise quality, and OVRL means overall quality.

| ID | Method | With Interference Without Interference SIG BAK OVRL SIG BAK OVRL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Noisy | 3.97 | 1.82 | 2.23 | 4.22 | 2.30 | 2.74 |
| 2 | NSNet2 [28] | 3.47 | 2.71 | 2.44 | 3.73 | 4.18 | 3.42 |
| 3 | DeepFilterNet2 [29] | 3.56 | 2.94 | 2.68 | 4.09 | 4.49 | 3.85 |
| 4 | TEA-PSE [15] | 3.69 | 3.64 | 3.16 | 4.16 | 4.44 | 3.89 |
| 5 | TEA-PSE2 [16] | 3.86 | 3.83 | 3.36 | 4.23 | 4.50 | 3.98 |
| 6 | BSRNN-S | 3.76 | 3.49 | 3.05 | 4.19 | 4.51 | 3.95 |
| 7 | pBSRNN-S | 3.66 | 3.91 | 3.21 | 4.17 | 4.50 | 3.94 |
| 8 | + MRSD | 3.90 | 3.81 | 3.37 | 4.30 | 4.50 | 4.04 |

Table 3.  Results of subjective listening, objective word accu- racy and final scores on the blind test set of DNS-2023.

| Method | Headset Speakerphone OVRL WACC SCORE OVRL WACC SCORE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Noisy | 1.22 | 0.843 | 0.449 | 1.24 | 0.857 | 0.459 |
| Baseline | 2.34 | 0.687 | 0.511 | 2.38 | 0.727 | 0.537 |
| pBSRNN-S + MRSD | 2.65 | 0.724 | 0.568 | 2.66 | 0.724 | 0.570 |

v.s.   18);  3) The use of MGD and MRSD training objectives can further improve the performance of the SE models. Models trained using MRSD achieves the best SE performance; 4) The best BSRNN model outperforms previous SE benchmark sys- tems on both offline (ID 5 v.s.  10) and online (ID 15 v.s.  20) scenarios and achieves the SOTA overall performance.

5.2.  Personalized speech-enhancement

In  Table  2,  we  present  the  PDNSMOS  ablation  results  of the proposed system on the DNS-2022 blind test set.   It can be observed that:   1) The non-personalized and personalized BSRNN models show comparable performance when there is no interfering  speech,  while  the personalized model  outper-

forms the non-personalized model for inputs without interfer- ence speech;  2) The use of MDR discriminator loss will de- grade the PDNSMOS score, one possible explanation is that the MGD loss is particularly designed for the PESQ-WB metric, which can be inconsistent with PDNSMOS; 3) After finetun- ing with the MRSD discriminator loss, both models can achieve better OVRL scores on the PDNSMOS metric; 4) The proposed model can achieve comparable or better results against previous SOTA PSE systems (ID 5 v.s.  10).

Table  3  shows  the  results  of  the  proposed  pBSRNN  in the DNS-2023 challenge.   We  found  that:   1)  The  proposed pBSRNN system obtains better performance against the base- line systems from the challenge organizer.  2) While the pro- posed pBSRNN system can significantly improve the subjec- tive OVRL scores, its use can largely reduce speech recognition word accuracy (WACC). This implies that current PSE mod- els may introduce unexpected artifacts and excessively suppress target speech when eliminating background noise and other in- terfering speech.

6.  Conclusion & Limitation

This paper presented the design of the BSRNN-based speech enhancement and personalized speech enhancement models and the corresponding training pipeline for full-band  speech  sig- nals.  With the modified band-level modeling strategy and the use of MGD and MRSD training objectives, the proposed sys- tems achieved competitive performance on various SE and PSE benchmark datasets.  However, as discussed in Section 6, the use of the SE model will severely degrade the speech recogni- tion performance, indicating that current SE and PSE models still suffer from over-suppression and model distortion. In addi- tion, we also find that when the timbres ofthe target speaker and the interfering speaker are similar, the model may fail to remove the interfering speech. Furthermore, if a channel mismatch oc- curs between the enrollment speech and the target speech, the model might mistakenly interpret them as distinct speakers.

7.  References

[1]  Y. Hu, Y. Liu, S. Lv, M. Xing, S. Zhang, Y. Fu, J. Wu, B. Zhang, and L. Xie, “Dccrn: Deep complex convolution recurrent network for phase-aware speech enhancement,” arXiv:2008.00264, 2020.

[2]  X. Hao, X. Su, R. Horaud, and X. Li, “Fullsubnet:  A full-band and sub-band fusion model for real-time single-channel speech enhancement,” in ICASSP 2021-2021 IEEE International  Con- ference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021, pp. 6633–6637.

[3]  Y. Luo and J. Yu, “Music source separation with band-split rnn,” arXiv:2209.15174, 2022.

[4]  A. Defossez, G. Synnaeve,  and Y. Adi,  “Real time speech en- hancement in the waveform domain,” arXiv:2006.12847, 2020.

[5]  A. Li, C. Zheng, L. Zhang, and X. Li, “Glance and gaze:  A col- laborative learning framework for single-channel speech enhance- ment,” Applied Acoustics, vol. 187, p. 108499, 2022.

[6]  K. Wang, B. He, and W.-P. Zhu, “Tstnn: Two-stage transformer based neural network for  speech enhancement in the time do- main,” in ICASSP 2021-2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP).   IEEE, 2021, pp. 7098–7102.

[7]  Y. Xia, S. Braun, C. K. Reddy, H. Dubey, R. Cutler, and I. Tashev, “Weighted speech distortion losses for neural-network-based real- time speech enhancement,” in ICASSP 2020-2020 IEEE Interna- tional  Conference on Acoustics, Speech and Signal Processing (ICASSP).    IEEE, 2020, pp. 871–875.

[8]  S.-W. Fu, C.-F. Liao, Y. Tsao, and S.-D. Lin, “MetricGAN: Gen- erative adversarial networks based black-box metric scores opti- mization for speech enhancement,” in International Conference on Machine Learning.    PMLR, 2019, pp. 2031–2041.

[9]  C. K. Reddy, H. Dubey, V. Gopal, R. Cutler, S. Braun, H. Gamper, R. Aichner, and S. Srinivasan, “Icassp 2021 deep noise suppres- sion challenge,” in ICASSP 2021-2021 IEEE International Con- ference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2021, pp. 6623–6627.

[10]  D. Harishchandra, A. Ashkan, G. Vishak, N. Babak, B. Sebastian, C. Ross, J. Alex, Z. Mehdi, T. Min, G. Hannes, G. Mehrsa, and A. Robert, “Icassp 2023 deep speech enhancement challenge,” in ICASSP 2022-2023.    IEEE, 2023.

[11]  S. Lv, Y. Fu, M. Xing, J. Sun, L. Xie, J. Huang, Y. Wang, and T. Yu, “S-dccrn: Super wide band dccrn with learnable complex feature for speech enhancement,” in ICASSP 2022-2022 IEEE In- ternational Conference on Acoustics, Speech and Signal Process- ing (ICASSP).    IEEE, 2022, pp. 7767–7771.

[12]  G. Zhang, L. Yu,  C. Wang,  and J. Wei,  “Multi-scale temporal frequency convolutional network with axial attention for speech enhancement,” in ICASSP 2022-2022 IEEE International  Con- ference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2022, pp. 9122–9126.

[13]  Q. Wang, H. Muckenhirn, K. Wilson, P. Sridhar, Z. Wu, J. Her- shey, R. A. Saurous, R. J. Weiss, Y. Jia, and I. L. Moreno, “Voice- filter: Targeted voice separation by speaker-conditioned spectro- gram masking,” arXiv preprint arXiv:1810.04826, 2018.

[14]  R. Giri,  S. Venkataramani, J.-M. Valin, U. Isik,  and A. Krish- naswamy,  “Personalized percepnet:  Real-time,  low-complexity target   voice   separation   and   enhancement,”    arXiv   preprint arXiv:2106.04129, 2021.

[15]  Y. Ju, W. Rao, X. Yan, Y. Fu, S. Lv, L. Cheng, Y. Wang, L. Xie, and S. Shang, “Tea-pse: Tencent-ethereal-audio-lab personalized speech enhancement system for icassp 2022 dns challenge,” in ICASSP 2022-2022 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP).   IEEE, 2022, pp. 9291– 9295.

[16]  Y. Ju, S. Zhang, W. Rao, Y. Wang, T. Yu, L. Xie, and S. Shang, “Tea-pse 2.0: Sub-band network for real-time personalized speech enhancement,” in 2022 IEEE Spoken Language Technology Work- shop (SLT).    IEEE, 2023, pp. 472–479.

[17]  S.-W. Fu, C. Yu, T.-A. Hsieh, P. Plantinga, M. Ravanelli, X. Lu, and Y. Tsao,  “MetricGAN+:  An improved version  of Metric- GAN for speech enhancement,” arXiv preprint arXiv:2104.03538, 2021.

[18]  W.  Jang,  D.  Lim,  J.  Yoon,  B.  Kim,   and  J.  Kim,   “Univnet: A  neural  vocoder  with  multi-resolution  spectrogram  discrimi- nators  for  high-fidelity  waveform  generation,”   arXiv  preprint arXiv:2106.07889, 2021.

[19]  J. Yu, H. Chen, Y. Luo, R. Gu, W. Li, and C. Weng, “Tspeech-ai system description to the 5th deep noise suppression (dns) chal- lenge,” in ICASSP 2023 -  2023 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 2023, pp. 1–2.

[20]  H. Wang,  C.  Liang,  S.  Wang,  Z.  Chen,  B. Zhang,  X.  Xiang, Y. Deng, and Y. Qian, “Wespeaker:  A research and production oriented speaker embedding learning toolkit,” in ICASSP 2023- 2023 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP).    IEEE, 2023, pp. 1–5.

[21]  X. Mao, Q. Li, H. Xie, R. Y. Lau, Z. Wang, and S. Paul Smolley, “Least squares generative adversarial networks,” in Proceedings of the IEEE international conference on computer vision, 2017, pp. 2794–2802.

[22]  C. Zheng, X. Peng, Y. Zhang, S. Srinivasan, and Y. Lu, “Inter- active speech and noise modeling for speech enhancement,” in Proceedings of the AAAI Conference on Artificial Intelligence, vol. 35, 2021, pp. 14 549–14 557.

[23]  F. Dang, H. Chen,  and P. Zhang,  “Dpt-fsnet:  Dual-path trans- former based full-band and sub-band fusion network for speech enhancement,” in ICASSP 2022-2022 IEEE International  Con- ference on Acoustics, Speech and Signal Processing (ICASSP). IEEE, 2022, pp. 6857–6861.

[24]  D. Yin, Z. Zhao, C. Tang, Z. Xiong, and C. Luo, “Tridentse: Guid- ing speech enhancement with 32 global tokens,” arXiv preprint arXiv:2210.12995, 2022.

[25]  A. Li, W. Liu, C. Zheng, C. Fan, and X. Li, “Two heads are bet- ter than one: A two-stage complex spectral mapping approach for monaural speech enhancement,” IEEE/ACM Transactions on Au- dio, Speech, and Language Processing, vol. 29, pp. 1829–1843, 2021.

[26]  A. Li, C. Zheng, Z. Zhang, and X. Li, “Mdnet: Learning monau- ral speech enhancement from deep prior gradient,” arXiv preprint arXiv:2203.07179, 2022.

[27]  S. Zhao, B. Ma, K. N. Watcharasupat, and W.-S. Gan, “Frcrn: Boosting feature representation using frequency recurrence for monaural speech enhancement,” in ICASSP 2022-2022 IEEE In- ternational Conference on Acoustics, Speech and Signal Process- ing (ICASSP).    IEEE, 2022, pp. 9281–9285.

[28]  S. Braun and I. Tashev, “Data augmentation and loss normaliza- tion for deep noise suppression,” in Speech and Computer: 22nd International Conference, SPECOM 2020, St. Petersburg, Russia, October 7—9, 2020, Proceedings 22.    Springer, 2020, pp. 79–86.

[29]  H. Schro… ter, A. Maier, A. Escalante-B, and T. Rosenkranz, “Deep- filternet2:  Towards real-time speech enhancement on embedded devices for full-band audio,” in 2022 International Workshop on Acoustic Signal Enhancement (IWAENC).    IEEE, 2022, pp. 1–5.

[30]  Y. N. Dauphin, A. Fan, M. Auli,  and D. Grangier,  “Language modeling with gated convolutional networks,”  in International conference on machine learning.    PMLR, 2017, pp. 933–941.

[31]  J. L. Ba, J. R. Kiros, and G. E. Hinton, “Layer normalization,” arXiv:1607.06450, 2016.

[32]  S. Ioffe and C. Szegedy, “Batch normalization: Accelerating deep network training by reducing internal covariate shift,” in Interna- tional conference on machine learning.     PMLR, 2015, pp. 448– 456.

[33]  Z. Chen, B. Liu, B. Han, L. Zhang, and Y. Qian, “The sjtu x-lance lab  system  for  cnsrc  2022,”  arXiv preprint  arXiv:2206.11699, 2022.